# Installation guide Nao and Pepper

This guide is to help you install all the necessary software to successfully work on the assignments of the HRI course. If you encounter any errors or other problems, contact Raymond (r.h.cuijpers@tue.nl). The files that are mentioned in this manual can all be found in Canvas, under the “Software” module.

Currently, there are different methods to access the robot:

- WinPython. Pre-installed with everything. It comes with the older Python 2.7 and works well with the robot, but is hopefully no longer needed.

- Python 2.7. Manual installation has many tiny version details.

- Choregraphe. Choregraphe is a graphical programming tool based on python 2.7. We use it for:

- Inspecting the state of the robot

- Manually controlling the robot

- Develop gestures

- Develop dialogs

- Use is as a robot simulator

# Winpython: Installing 32-bit version of Python 2.7

To be able to communicate with the simulated (or real) NAO-robot you need to have the 32-bit version of Python 2.7. There are two ways of installing this: through WinPython or manually.

### Method 1)

- Download “WinPython-32bit-2.7.10.3.exe” package*

- Extract to folder c:\developer (or equivalent)**

- Open the WinPython subfolder and Run Idle to start programming

- Optionally, you can register this distribution to make it visible to other programs (see step 3 and 4) of method 2)

- Optionally, install additional libraries:

  - Open the “Winpython Command Prompt.exe”. Use search (Windows + S) and type the name.

  - Check whether you have the correct version of python, type:
python --version

  - Type python -m pip install opencv-python==4.2.0.32 (this is the last version of opencv that support python 2.7

(*) For recent WinPython, Windows users may have to install missing system DLL Microsoft Visual C++ Redistributable for Visual Studio 2017..2019 (vc_redist_x86.exe for WinPython 32bit, vc_redist_x64.exe for Winpython 64bit)

(**) For best Winpython 2018-01 and later experience, it is recommended to have WinPython base directory path smaller than 37 characters. example: C:\Users\xxxxxxxx\Downloads\WinPython

### Method 2) – Install WinPython manually

- Install the 32-bit version of WinPython 2.7 and install it. See details here: https://sourceforge.net/p/winpython/wiki/Installation/

- Go to the new WinPython folder with “IDLE (Python GUI).exe” in it.

- Run “WinPython Control Panel.exe”.

- Select tab “advanced”, select “register distribution” and click “yes”.

- Proceed to manual installation of naoqi SDK below

- Install additional libraries:

  - Open the “Winpython Command Prompt.exe”. Use search (Windows + S) and type the name.

  - Check whether you have the correct version of python, type:
python --version

  - Type python -m pip install opencv-python==4.2.0.32 (this is the last version of opencv that support python 2.7

The big advantage of using WinPython is that it can run simultaneously next to a different Python-distribution that might be installed on your computer, e.g. a later version of Python.

Note: DO NOT USE “pip install --upgrade pip”. This will break pip. Instead use “easy_install pip==20”. This will also fix a broken pip.

To install everything do:

easy_install pip==20

pip install --upgrade setuptools wheel

pip install numpy scipy matplotlib

- On windows it is common to precede the pip command by python -m, e.g.:

python -m pip install numpy scipy matplotlib (newer windows distributions use py instead of python)

- It is always a good idea to check the version of your python distribution, e.g.: 
python –version ( or py –version on windows) might point to python 2.x or python 3.x depending on your installation process.

Optionally add opencv:

pip install opencv-python==4.2.0.32

Optionally,  add python text-to-speech (https://pypi.org/project/pyttsx3/)

It is fine if this doesn’t work, it is just convenient for some tests, but not necessary.

pip install pywin32==232

pip install pyttsx3

Keep the F.A.Q. from canvas at hand as it contains solutions to many of the frequent problems

### Method 3) Manual installation from Python.org

The other option is to manually install the 32-bit version of Python 2.7.

- On the Python website (https://www.python.org/downloads/) all the versions can be found. Make sure to choose a version of Python 2.7 and choose the 32-bit version.

- It is strongly recommended to use virtual environments for this method (see e.g. https://packaging.python.org/en/latest/guides/installing-using-pip-and-virtual-environments/ ).

- After installation follow the steps of Method 2) – Install WinPython manually to install the packages. For general instructions for installing packages, see below.

#### Python Code editor

In both options, the default code editor is IDLE (Python GUI), and it is automatically installed with the WinPython installation. It is possible to use your own editor, if preferred (PyCharm, Spyder, Visual Studio Code). Microsoft’s Visual Studio Code is recommended, especially if you have other Python distributions installed on your computer

# Installing Python Packages on Python2.7

In both options it might be necessary to install some packages. In WinPython scientific packages such as NumPy and SciPy are already installed, but in the manual version they are not installed automatically. Other packages such as pyttsx3, pyaudio and speechrecognition are optional. Luckily Python made this easy: you can use PIP for this as described above.

## For WinPython:

Run “WinPython Command Prompt.exe” which can be found in the folder of WinPython.

For manually installed Python versions: 
Open the regular command shell by typing CMD in the search option

In the opened command shell type: python -m pip install package_name ( or py -m install on newer python distributions on windows)

Replace package_name by the name of the package you are trying to install (numpy, scipy, pyttsx3, etc.)

Note: for pyttsx3 first update pywin32 to version 223 using “pip install winpy32==223”. Newer versions will break pyttsx3.init() command in Python.

If you installed the manual version it is always a good idea to install numpy, scipy and matplotlib. For example:

pip install numpy scipy matplotlib

pip install opencv-python==4.2.0.32

After installing a package you can test whether it worked by opening IDLE (Python GUI) and type:

>>> import numpy
>>> import scipy
>>> import cv2
Etc.

These commands should give no error.

# Installing naoqi SDK for Python 2.7

To be able to communicate with the simulated (or real) NAO robot, you need to have the Python SDK of naoqi (the OS of the NAO robot). There used to be an installer, but it assumes a system wide installation of python 2.7, which is usually not the case.

- Download “pynaoqi-python2.7-2.5.7.1-win32-vs2013.zip”

- Extract in a suitable folder e.g. c:\development\

- Make naoqi library findable for Python. There are many ways:

  - Deprecated Adding the path tot the PYTHONPATH environment variable. Not recommended if you have multiple python versions. See installing python above for how to add environment variables. You will need to restart python and your command shell to make this work.

  - Add to sys.path inside of your own script:

import sys
sys.path.append(r“c:\development\pynaoqi-python2.7-2.5.7.1-win32-vs2013\lib”)

Note that you need to either escape the backslash character by using \\ or use a raw string by preceding it with r. Using slash also works.
Note that at the end it says “\lib”. Don’t forget!

  - Create a naoqi.pth file inside the site-packges directory of your python distribution

    - Find the site-packages directory in your python distribution e.g. c:\development\Python27\lib\site-packages

    - Create a file called naoqi.pth containing only the path to your pynaoqi library:

c:\\development\\pynaoqi-python2.7-2.5.7.1-win32-vs2013\\lib

Method b is most flexible, method c is more permanent

- Check your installation by opening IDLE (Python GUI) and check you are running the right python version. Then type: 

>>> import naoqi

If this gives no error, the naoqi SDK is installed correctly. If it does give an error, it usually means the pathname is not added correctly

To test and use naoqi, download and run test_qi.py and/or test_naoqi.py. They provide excellent examples on how to use qi and naoqi directly. You can find them on canvas or on https://github.com/Rctue/nao-lib

## Using our nao-lib library

To assist ourselves and students in generating code for the NAO robot, we created a Python NAO-library. The library is designed for the real robot, but can also be used for the simulated robot in Choregraphe. All the versions with examples can be found on (https://github.com/Rctue/nao-lib).

The latest version is nao_nocv_2_1.py, which comes pre-installed with WinPython. If you want to use it in another distribution follow these guidelines:

- Download nao_nocv_2_1.py from canvas or github, and put it in the same folder as your python project files for the course.

- Type:
>>> import nao_nocv_2_1

## Testing the nao-lib library

Testing the nao-lib library involves knowing what functions are present and how they (are supposed to) work. This is not the goal, however. We just want to test simple functionality. There are multiple ways:

- Run one of the test_naolib files from https://github.com/Rctue/nao-lib. These files are excellent examples of how to use nao-lib

- Create a new Python file.

  - Create a file called test.py with the following content:

import nao_nocv_2_1 as nao #with this line you import all the functions of the naolibrary and are able to call them with “nao”

nao.InitProxy(“127.0.0.1”) #with this line you initialize all the proxies, and indicate the IP-adres. In this case this is 127.0.0.1, because this is your home ip-adress.

nao.InitPose() #this line makes the robot stand up

nao.Walk(1,0,0) #this line makes the robot walk 1 meter in x direction

nao.Crouch() #this line makes the robot crouch

  - Run the Python code test.py after adjusting the IP address and Port number to the correct values (typically 192.168.0.100 and 9559).

# Installing Choregraphe

Choregraphe is software designed to program the NAO and Pepper robot. It is an intuitive way of programming, using boxes that connect to each other. It is limited, which is why we regularly use Python instead of Choregraphe to program our robots. However, Choregraphe is a useful tool for developing dialogs and gestures.

To install Choregraphe you have to download it from canvas.

- Download Choregraphe. Choregraphe 2.8.x is used for the Robbie (black) nao robot. The version of Choregraphe for the Bender (orange) nao robot and Eve (pepper) is 2.5.

- Run the installation file

- Copy the key from the License_key.rtf on Canvas and enter it when you first run the program.

## Choregraphe: Testing with a simulated robot

It is also possible to test everything without the real robot. To do so, you first need to start a virtual robot. The easiest way to do that is by using Choregraphe.

- Open Choregraphe and wait until a robot appears in the robot view. If that fails, disconnect and select “connect to a virtual robot” from the menu. Make note of the IP number and Port number:

- Go to Edit/Preferences menu, click on the Virtual robot tab, Note the Port number in green

- Click on the connect icon and make note of the IP address and Port number, or right-click on the robot and copy it.

- Run the Python code test_qi.py (or any other program) after adjusting the IP address and Port number to the correct values (typically 127.0.0.1 and the port number noted above).

- The simulated robot should move and talk (text balloons).

Note: You will get some errors because the simulated robot does not have all the proxies of the real robot (simulated robot has no ALAudioDevice, ALFaceDetection, ALFaceTracker, ALSpeechRecognition, ALVideoRecorder, ALLandmarkdetection, ALSoundDectection and ALAudioSourceLocalization). This is not a problem. If you get other errors and the simulated robot does not move, you are probably not connected to the simulated robot. Check the IP-adress and the port number of the simulated robot in Choregraphe.

Note: Choregraphe and Python need access to the network. Make sure you give access when your OS asks to whether you want to give permission.