# Installation guide Nao and Pepper

This guide is to help you install all the necessary software to successfully work on the assignments of the HRI course. If you encounter any errors or other problems, contact Raymond (r.h.cuijpers@tue.nl). The files that are mentioned in this manual can all be found in Canvas, under the “Software” module.

Currently, there are three methods to access the robot:

- Python 3.x. The newest Python 3 method is better, but is still under development.

- WinPython. Pre-installed with everything. It comes with the older Python 2.7 and works well with the robot, but is hopefully no longer needed. Installation is described in separate document for python 2.7.

- Choregraphe. Choregraphe is a graphical programming tool based on python 2.7. We use it for:

- Inspecting the state of the robot

- Manually controlling the robot

- Develop gestures

- Develop dialogs

- Use is as a robot simulator

# Installing Python 3.x

Python 3 uses libqi to communicate with the robot, which is easier and more versatile than the older naoqi.

After installing python 3.11 or higher, go to your python distribution and create a virtual environment

- Go to your command shell and type python –version and check the version is >=3.11

- Create a virtual environment https://www.w3schools.com/python/python_virtualenv.asp and activate it. If you are using anaconda use conda instead https://docs.conda.io/projects/conda/en/latest/user-guide/tasks/manage-environments.html

- Install python packages: numpy, scipy, matplotlib, opencv and libqi-windows. Libraries are correctly installed if you can import them from Python e.g. import numpy as np.

pip install numpy scipy matplotlib

pip install opencv-python

pip install libqi-windows

Note: On windows you sometimes need to type python -m pip install <packagename>

Note: On windows you may need to use py3 instead of python

## Verifying libqi with robot

import qi

print(qi.__version__)  # 3.1.5

session = qi.Session()

session.connect("tcp://nao.local:9559")

tts = session.service("ALTextToSpeech")

tts.say("Hello from Windows!")

More advanced test is available using the test_qi.py file available in the file section on canvas or from github: https://github.com/Rctue/nao-lib .

# Installing Choregraphe

Choregraphe is software designed to program the NAO and Pepper robot. It is an intuitive way of programming, using boxes that connect to each other. It is limited, which is why we regularly use Python instead of Choregraphe to program our robots. However, Choregraphe is a useful tool for developing dialogs and gestures.

To install Choregraphe you have to download it from canvas.

- Download Choregraphe. Choregraphe 2.8.x is used for the Robbie (black) nao robot. The version of Choregraphe for the Bender (orange) nao robot and Eve (pepper) is 2.5.

- Run the installation file

- Copy the key from the License_key.rtf on Canvas and enter it when you first run the program.

# Choregraphe: Testing with a simulated robot

It is also possible to test everything without the real robot. To do so, you first need to start a virtual robot. The easiest way to do that is by using Choregraphe.

- Open Choregraphe and wait until a robot appears in the robot view. If that fails, disconnect and select “connect to a virtual robot” from the menu. Make note of the IP number and Port number:

- Go to Edit/Preferences menu, click on the Virtual robot tab, Note the Port number in green

- Click on the connect icon and make note of the IP address and Port number, or right-click on the robot and copy it.

- Run the Python code test_qi.py (or any other program) after adjusting the IP address and Port number to the correct values (typically 127.0.0.1 and the port number noted above).

- The simulated robot should move and talk (text balloons).

Note: You will get some errors because the simulated robot does not have all the proxies of the real robot (simulated robot has no ALAudioDevice, ALFaceDetection, ALFaceTracker, ALSpeechRecognition, ALVideoRecorder, ALLandmarkdetection, ALSoundDectection and ALAudioSourceLocalization). This is not a problem. If you get other errors and the simulated robot does not move, you are probably not connected to the simulated robot. Check the IP-adress and the port number of the simulated robot in Choregraphe.

Note: Choregraphe and Python need access to the network. Make sure you give access when your OS asks to whether you want to give permission.

# Misty Robot

To access the misty robot from python you need to install Misty-SDK for Python version 3.8 or higher. Note that this is not Python 2.7, which was needed for the Nao and Pepper robot. So you need to use a Python 3.8 or higher installation and create a separate virtual environment (see e.g. https://packaging.python.org/en/latest/guides/installing-using-pip-and-virtual-environments/ ).

Open the command shell for your Python 3 distribution and install mistyPy:

pip install git+https://github.com/MistyCommunity/Python-SDK.git

• Do not use “pip install mistyPy”, it is outdated

Details on how to use this package can be found here: https://github.com/MistyCommunity/Python-SDK

As for the nao robot you should install other useful libraries as needed:

pip install numpy scipy matplotlib

pip install opencv-python